# PreLab 05 – Worksheet

| Name: Mohammad Husain Naqvi (11062) Syed Fakhar Iqbal Rizvi | ID: 11062 11091 | Section: T-4 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

No, the above code is not synthesizable due to the following reasons: 1. ‘sum’ not being initialized in the loop. 2. The variable n is also not provided with any value or constant. Corrected code: parameter n = 4; integer i; always @(*) begin sum = 0; // Initialize to prevent latch inference for (i = 0; i < n; i = i + 1) begin sum = sum + a[i]; end end

## Task 2

Fixed Code: initial begin clk = 0; // Initialize to define the starting state end always begin #10 clk = ~clk; // Period = 20 time units (50 MHz if 1 ns timescale) end

## Task 3

This code basically describes a positive edge triggered D flip flop. With an enable variable ‘ en ‘. q and d have a non-blocking assignment between them. - When en = 1 and clock is on positive edge. q will get the value of d. - When en = 0 and clock is on positive edge. q will not get the value of d and remain in their current state.